Document Revision Tracking

| Issue Date | Notes |
| --- | --- |
| November 2021 | First Publication |

Contents

Introduction	4

1.	AIR Overview	6

1.1	General Information	6

1.1.1	Asset Information Management Functions	6

1.1.2	Asset Information Management Systems	7

1.2	Quality Assurance Procedures	7

1.2.1	Quality Systems	7

1.2.2	Responsibility	8

2.	Asset Information Requirements	9

2.1	AIR delivery formats	9

2.2	Model breakdown structure for Asset Management	9

2.3	Systems breakdown Information	9

2.4	Nomenclature	10

2.5	BIM Object	10

2.5.1	Library requirements	10

2.5.2	Object Library Types	11

2.5.3	Symbology requirements	11

2.6	Classification	11

2.7	Schedule of Asset	11

2.8	Accuracy of Information	11

2.9	Schedule of Information Requirements	12

2.10	Key Compliance Requirements	13

3.	COBie	14

3.1	Constructions Operations Building information exchange requirements format	14

3.2	Constructions Operations Building information exchange requirements	15

3.3	COBie Deliverable requirements	15

3.3.1	COBie Schema Requirements	15

3.4	COBie Contacts	16

3.5	Facility		17

3.6	Floor	18

3.7	Space	18

3.8	Zone	19

3.9	Asset Type	20

3.10	Asset Component	21

3.11	Asset System	23

3.12	Assembly	23

3.13	Spare	24

3.14	Resource	24

3.15	Job	25

3.16	Document	27

3.17	Attribute	28

3.18	Issue	29

3.19	Delivery of COBie	29

3.20	Further COBie Information	30

Introduction

Asset Information Requirements

Asset Information Requirements (AIR) should provide the basis for the commercial, managerial, and technical aspects of producing and managing asset information and should include the standards, methods, and procedures to be implemented by the Delivery Team. The AIR should provide the detailed specification for the delivery of the Asset Information Model (AIM) and take on board the considerations identified under Section 2.16 on AIM.

The AIR should include the detailed asset information deliverables required to answer the asset related OIR. The AIR should also be expressed in a manner to allow inclusion in asset management appointments to support better organisational decisions. The AIR should facilitate the cascading of requirements across multiple appointments and the amalgamation of data deliverables to provide a consolidated response to the OIR.

The AIR needs to specify precise information for each asset, whether that be an entire building, a zone, space, or individual object (component), such as a door or piece of furniture. As a minimum the AIRs should include regulatory requirements, health and safety information, security, operations and maintenance (O&M) aspects, as well as provide a unique identifier for each asset type. The use of identified classifications systems and schema associated with assets will facilitate the cascading of requirements.

An individual product category Asset Information Requirement is best delivered as a data template and is equally applicable to products as well as spaces, zones, or an entire building. The development of standardised data dictionaries to hold and manage data requirements facilitates enhanced information management, e.g., by allowing standardised document delivery through reporting in open industry formats such as OpenOffice, but also digital formats such as JSON and XML via open application programming interfaces (APIs). This in turn can facilitate automated verification of deliverables against requirements.

Delivery of the AIR against each project milestone should be established within the level of information need. The format of delivery of the AIR should also be clearly defined and established in the EIR. Model information can be provided by open data format options, such as the Construction Operations Building Information Exchange (COBie) and IFC 2X3 TC1 using the Basic FM Handover View. Manufacturers and suppliers can also provide standardised product data sheets based upon industry templates which could in turn be accessible via URLs, QR codes, Bar codes or RFID tags.

The Scope

The Asset Information Requirements (AIR) will include the following elements:

- Detailed Specification for delivery of Asset Information Model

- AIR Deliverables

- COBie

- Data Template requirements

The purposes of these documents are to clearly define the Appointing Party / Clients information requirements regarding all Assets that are owned and managed by the Appointing Party / Client and associated information in order that the Appointing Party / Client can procure a lifecycle data approach to information management.

The Solution

The solution to the requirement is to continuously review, update and address any changing requirements through the course of the project.

This document has been produced to enable all delivery team members to work in a consistent manner and ensure that information is delivered and verified against the requirements identified. The project must support the aggregation of this information required all parties.

Schedule of Requirements and Supporting document

This document, the Asset Information Requirements shall be read in accordance with the following schedule of documents which make up the Appointing Party / Clients information requirements and brief:

| Document Name | Revision | Status |
| --- | --- | --- |
| Organisational information requirements |  |  |
| Asset information requirements |  |  |
| Project information requirements |  |  |
| Security information requirements |  |  |
| Exchange information requirements |  |  |
| Project information standards |  |  |
| Project information production methods and procedures |  |  |

AIR Overview

The asset information requirements are a subset of the Appointing Party / Clients information requirements which in turn are a subset of the Appointing Party / Clients brief. The purpose of Asset Information requirements is to identify the requirements for Asset Data using an approach that can be verified and aid in reducing the waste identified in creating mountains of irrelevant, ambiguous and unreliable information in a format that is difficult to use or find information.

The Asset information requirements set out the technical aspects of the information to be provided at handover from the construction team to support operations and maintenance activities in a manner that can be verified.

General Information

This section sets out the people, technology and processes required for the asset information requirements. These AIRs should be read in conjunction with the following Exchange Information Requirements (EIR):

| Name | Document |
| --- | --- |
| Appointing Party / Clients EIR | CIC BIM Exchange Information Requirements (EIR) Template |

(To be completed by the business’s representative.)

Asset Information Management Functions

This project Asset Information Management functions identifies the roles and responsibilities relating to this project. These will be undertaken by the following:

| Function | Name | Company | Contact Details |
| --- | --- | --- | --- |
| Appointing Party / Client Delivery Manager |  |  |  |
| Appointing Party / Client Information Manager |  |  |  |
| Asset Manager |  |  |  |
| Operations Manager |  |  |  |
| Asset Security Manager |  |  |  |

Asset Information Management Systems

The following technologies are the current / proposed asset, operations, and facilities management systems to be used, along with the acceptable data formats:

| Software application | Software Use | Software vendor | Package / Version | File formats for delivery |
| --- | --- | --- | --- | --- |
| XYZ | Facilities management Operational |  |  | COBie |
| ABC | Asset Maintenance |  |  |  |
| 123 | Room bookings |  |  |  |

Quality Assurance Procedures

Quality Systems

The following processes and quality systems are in place to support the definition and delivery of Asset Information. This document must be delivered in accordance with the requirements of the following:

| Quality Procedures | Documentation document name |
| --- | --- |
| Asset Information verification process |  |
| COBie validation procedure |  |
| Handover Procedure |  |
| Asset Coding and numbering |  |

Responsibility

The following identified responsibility have been identified to provide asset data and information for the project as follows:

| Identified responsibilities (using RACI) R = Responsible A = Accountable C = Consulted I = Informed | Lead Designer | Designer | Main Contractor | Sub-Contractor / Installer | Manufacturer |
| --- | --- | --- | --- | --- | --- |
| Design and performance criteria for each work package | R |  |  |  |  |
| Coordinated design model |  | R |  |  |  |
| Coordinated construction model |  |  | R |  |  |
| As Built model delivery |  |  | R |  |  |
| Provide digital O&M |  |  | R |  |  |
| Provide product data |  |  |  |  | R |
| Provide warranty information, maintenance and operational requirements and spares |  |  |  | R |  |
| COBie data |  |  |  |  |  |

Asset Information Requirements

The delivery of Asset information requirements is fundamental to providing the lifecycle approach of information from Designers and Constructors for use by operators and maintainers. This section sets out the detailed requirements for delivery of information for the project.

AIR delivery formats

The delivery of asset information will be provided in the following formats:

| Information Type | Format | Version / build |
| --- | --- | --- |
| Models | .rvt | R2022 |
| Drawings | .dwg / .pdf | R2022, PDF 2.0 |
| Schedule | .xlsx | Excel 2019 |
| Product Data | .json | JavaScript |

Model breakdown structure for Asset Management

To maintain and manage the asset the information must be broken down using the following work breakdown structure and federation strategy to facilitate effective maintenance and operations:

| Req. | Element | Model(s) | Discipline(s) |
| --- | --- | --- | --- |
| ☒ | Structure, Shell, and Core | Substructure, structure, façade | Structure / Architecture |
| ☐ | Fixture and fittings |  |  |
| ☐ | Loose fittings |  |  |
| ☐ | Drainage |  |  |
| ☐ | HVAC |  |  |
| ☐ |  |  |  |

Systems breakdown Information

To maintain and manage the asset each System and associated circuit must be identifiable and labelled using the following criteria:

| Req. | System name | System ID | Circuit IDs | Nomenclature |
| --- | --- | --- | --- | --- |
| ☐ | HVAC Supply |  |  |  |
| ☐ | HVAC Return |  |  |  |
| ☐ | HVAC Intake |  |  |  |
| ☐ | Drainage Soil, below ground |  |  |  |
| ☐ | Drainage Surface Water, below ground |  |  |  |

Nomenclature

The following Nomenclature must be adopted to facilitate the interoperability of information between project and asset management. The Nomenclature covers file unique identifiers, BIM objects and Asset Coding requirements. Full details of these should be set out within the Project Information Standards. The items identified are as follows:

| Item | Nomenclature | Example |
| --- | --- | --- |
| File Unique ID | (Hong Kong ‘Local Annex’ of ISO 19650-2:2018) | (BIMS2020-CIC-ZZ-01-AR_AA-M3-001) |
| BIM object naming |  |  |
| Asset Unique ID |  |  |
| Systems |  |  |
| Facility |  |  |
| Levels |  |  |
| Zones |  |  |
| Spaces |  |  |
| Rooms |  |  |
| Windows |  |  |
| Doors |  |  |

BIM Object

To facilitate consistent delivery for Asset Management BIM Object should be generated considering the following requirements:

| Requirements | Reference |
| --- | --- |
| BIM Object Standard | CIC Production of BIM Object Guide - General Requirements (August 2019) or later version |

Library requirements

To facilitate the delivery of structured information to meet Asset requirements existing BIM Objects may be used from the following Library source:

| Requirements | Reference |
| --- | --- |
| BIM Object Source | (BIM Objects ¦ Resources ¦ BIM (cic.hk)) |
| BIM Object Library |  |

Object Library Types

It should be noted that unique object types must be created within the BIM model to reflect the type information requirements for Asset Management. Each manufactured product must be identified by a different BIM object Type and NOT by the application of properties at instance level. Clarity regarding whether information is to be provided based upon instance or type-based properties must be established at the initial stage.

Symbology requirements

The use of the following symbology standards shall be adopted in all BIM / CAD information. Each project shall include a KEY drawing confirming all symbology used not indicated below:

| Object | Reference |
| --- | --- |
| Architectural Symbols | BS 8541-2:2011 |
| Electrical Symbols |  |
| Mechanical Symbols |  |

Classification

All elements shall be classified in accordance with the OmniClass classification and shall follow as indicated below:

| Objects | Table | Description |
| --- | --- | --- |
| Entities | Table 11 | Construction Entities by Function |
| Spaces | Table 14 | Space by Form |
| Works Results | Table 22 | Results |
| Products | Table 23 | Products |

Schedule of Asset

To clearly identify information requirements associated with each asset type the Appointing Party / Client shall provide a schedule of assets along with an associated product data template as identified below:

| Asset | Classification | Product Data Template (Asset Template with list of properties or attributes) |
| --- | --- | --- |
| Actuator | 23-27 33 00 | BIM-AM MEP Asset Template |
| Air Blower | 23-33 31 13 11 | BIM-AM MEP Asset Template |
| Air Compressor | 23-27 21 00 | BIM-AM MEP Asset Template |

Accuracy of Information

To ensure effective asset management, it is expected that all model graphical information supplied is drawn graphically and dimensionally correct to a scale of 1:1. The permitted tolerances for survey and as built information shall be as follows:

| Item | Tolerance |
| --- | --- |
| Building structure | +/- 5mm in 1000mm |
| External items | +/- 10mm in 1000mm |

Upon completion of any works an accurate as built model will be provided four weeks prior to handover of the works.

Schedule of Information Requirements

According to ISO 19650, AIR set out three aspects of asset information:

- Managerial: Defines the information standards and production methods and procedures to be used by the delivery team.

- Commercial: Defines the information standards and production methods and procedures to be used by the delivery team.

- Technical: Details the information specifications to meet the organisational information requirements relevant to the asset.

(Type of Information,	Examples of Information

Managerial

- Type of asset

- Photograph

- Identification numbers (specification number, product number, serial number)

- Location

- Floor area

- Warranties and guarantee periods

- Access planning and work schedules

- Maintenance and inspection schedules

- Outstanding tasks

- Record of planned and unplanned maintenance

- Standards, processes, and procedures

- Hazardous contents or waste

- End of life processes

- Emergency plans

Commercial

- Description

- Function

- Details of supplier

- Lead time

- Condition

- Key performance indicators

- Performance targets or standards

- Non-conformance criteria and actions to be taken

- Criticality to the organisation

- Details of spares

Technical

- Engineering data

- Design parameters

- Interdependencies

- Commissioning dates and data

- Operational data

- Finishes

- Services requirements

- Outputs

Financial and

- Original cost

- Operating cost

- Planned maintenance cost

- Historical maintenance cost

- Replacement value

- Downtime impact

Legal

- Ownership

- Maintenance demarcation

- Work instructions

- Contractual information

- Risk assessments and control measures)

Additional requirements for asset information are identified in the following locations:

| Requirement | Location |
| --- | --- |
| Point cloud data | cloud hosting location |
| Photogrammetry | cloud hosting location |
| Product Data library | cloud hosting location |
| Legal requirements | cloud hosting location |
| Cost data | cloud hosting location |

Key Compliance Requirements

The following table sets out the key compliance requirements for delivery of all Asset Information from the identified project as follows:

| Requirement | Delivery Format |
| --- | --- |
| BIM Model validation criteria | .rvt / .nwd |
| Document validation criteria | .pdf |
| COBie validation criteria | .xls |

COBie

The delivery of Constructions Operations Building information exchange (COBie) is an internationally agreed format for the delivery of Asset information from Designers and Constructors for use by operators and maintainers. This section sets out the detailed requirements for delivery of COBie for the project

(The content of the COBie section can be delivered in different formats and this section identifies the appropriate approach that can be made. Alternatively information could be provided through Asset Information Lists or through other open formats such as Industry Foundation Classes.)

The following statements set out the requirements, context and intended use for information received, processed and to APPOINTING PARTY / CLIENT NAME, to support the delivery of its strategic objectives:

- To enable a Health and Safety approach with access to Health and Safety plus associated Risk management information using a digital approach

- To facilitate the ability to access correct and current information within the Project information Model

- To have the ability to verify the digital Operations and Maintenance information prior to project handover in accordance with the Asset information requirements (AIR)

- To be able to identify all construction and manufacturers warranties, including warranty periods and maintenance requirements for all maintainable assets

- To be able to identify the end of viable or economic life of any assets or asset systems along with replacement costs

- To have access to appropriates spares and asset replacement requirements for building assets or components

- To reduce remedial costs associated with handover and early operation relating to construction / installation / testing / assembly / commissioning and training defects

- To remove the need for supplementary surveys to validate project information or fill gaps with information provided at handover:

- To have data in an open and interoperable format to utilise the data in range of databases (COBie BS1192:4)

- To reduce the manual processing of building information / data at handover.

- To be able to access consistent gross internal / useable area data for all buildings.

- To help reduce the cost of maintaining specific assets / asset systems through earlier analysis with FM

Constructions Operations Building information exchange requirements format

The delivery of COBie information will be provided in the following format:

| COBie Format | Version |
| --- | --- |
| COBie XML |  |
| .XLS |  |
| COBieLite |  |

Constructions Operations Building information exchange requirements

The following Table sets out the COBie structure and identifies what information delivery are a project requirement and at what stage:

| Required | COBie Tab Name | Project Stage(s) | Detailed requirements (reference) |
| --- | --- | --- | --- |
| ☐ | Facility |  |  |
| ☐ | Contact |  |  |
| ☐ | Floor |  |  |
| ☐ | Space |  |  |
| ☐ | Zone |  |  |
| ☐ | Asset Type |  |  |
| ☐ | Asset Component |  |  |
| ☐ | Asset System |  |  |
| ☐ | Assembly |  |  |
| ☐ | Spare |  |  |
| ☐ | Resources |  |  |
| ☐ | Job |  |  |
| ☐ | Document |  |  |
| ☐ | Attribute |  |  |
| ☐ | Issue |  |  |
| ☐ | Other |  |  |

COBie Deliverable requirements

To facilitate an effective information exchange, the delivery team will provide the following COBie information as identified 4 weeks prior to handover of the built asset.

COBie Schema Requirements

The following colour schemes are used to identify the source and delivery requirements using COBie as identified:

COBie Colour Coding

Required

Reference Sheet or Pick List

Required if linked with Application

Required if specified

COBie Contacts

The following table identifies the COBie Contact information required to be delivered

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☐ | Email | Suppliers Contact Email | (abc@mail.com) |
| ☒ | CreatedBy | Supplier who created Record | (abc@mail.com) |
| ☒ | CreatedOn | Date record was created | (2020‐01‐15T14:30:00) |
| ☒ | Category | OmniClass Code |  |
| ☒ | Company | Suppliers Company Name | (Company |
| ☒ | Phone | Suppliers Company Phone Number | (+852 0000 0000) |
| ☐ | ExternalSystem | The External System for the Contact from which the data came. |  |
| ☐ | ExternalObject | The External Object of the System for the Contact from which the data came. |  |
| ☐ | ExternalIdentifier | The External Identifier of the System for the Contact from which the data came |  |
| ☐ | Department | The department the contact belongs to with the Company. | (Design) |
| ☐ | OrganisationCode | The Code shorthand used by the Appointing Party / Client to recognise the company as per CIC BIM Standards (General) | (ABC) |
| ☐ | GivenName | The Contact Given name, First name, Christian name. | (Given Name) |
| ☐ | FamilyName | The Family Name, Surname of the Contact. | (Family Name) |
| ☒ | Street | The street name. This should include building number and any estate, or locality that helps identify the address. |  |
| ☒ | PostalBox | The postal box of contact address. |  |
| ☒ | Town | The Town of the address of the contact. | (Town Name) |
| ☒ | StateRegion | The State, Region, County of the contact address. |  |
| ☒ | PostalCode | The postal code of contact address as used by the national postal service of the country. | (Zip / Post Code) |
| ☒ | Country | The country name to the standard of ISO 3166-1 English short name case sensitive. |  |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Accommodation Schedule

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName

Facility

The following table identifies the COBie Facility information required to be delivered:

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | A Unique Code for the project to be consistent with all other Cobie files for the project. | 127 |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | The facility will be assigned with a classification category |  |
| ☒ | ProjectName | Project Code following CIC BIM Standard (General) | ABC |
| ☒ | SiteName | Site Code following CIC BIM Standard (General) | (SC) |
| ☒ | LinearUnits | The linear units to be used throughout the facility. | (millimetres) |
| ☒ | AreaUnits | The area units to be used throughout the facility. | (Metres squared) |
| ☒ | VolumeUnits | The volume units to be used throughout the facility. | (Metres cubed) |
| ☒ | CurrencyUnit | Project specific units | (Hong Kong Dollar) |
| ☒ | AreaMeasurement | The area measurement units to be used throughout the facility. | Authoring default |
| ☐ | ExternalSystem | The External System for the Facility from which the data came. |  |
| ☐ | ExternalProjectObject | The External Object of the System for the Project from which the data came. |  |
| ☐ | ExternalProjectIdentifier | The External Identifier of the System for the Project from which the data came |  |
| ☐ | ExternalSiteObject | The External Object of the System for the Site from which the data came. |  |
| ☐ | ExternalSiteIdentifier | The External Identifier of the System for the Site from which the data came |  |
| ☐ | ExternalFacilityObject | The External Object of the System for the Facility from which the data came. |  |
| ☐ | ExternalFacilityIdentifier | The External Identifier of the System for the Facility from which the data came |  |
| ☐ | Description | A description of the facility its purpose and ethos. | Ancillary Building |
| ☐ | ProjectDescription | The project description if this is different from the main description. | (Project XXX) |
| ☐ | SiteDescription | The site description. |  |
| ☐ | Phase | The current phase of the project. The phases should be from a recognised industry categorisation. | (Stage 5) |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Accommodation Schedule

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName

Floor

Identify the following COBie Facility information required to be delivered:

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Floor Name | (1F) |
| ☒ | CreatedBy | Supplier who created Record | (abc@mail.com) |
| ☒ | CreatedOn | Date record was created | (2020‐01‐15T14:30:00) |
| ☒ | Category | Floor, Roof or Slab following BS1192:4 guidance | (Floor) |
| ☐ | ExtSystem | The External System for the Floor from which the data came. |  |
| ☐ | ExtObject | The External Object of the System for the Floor from which the data came. |  |
| ☐ | ExtIdentifier | The External Identifier of the System for the Floor from which the data came |  |
| ☒ | Description | A description of the floor, site, roof. | (First Floor) |
| ☒ | Elevation | The level of the floor from the base datum as a numeric value in the unit type as defined in the Cobie.Facility.LinearUnits for the project. | (1000) |
| ☒ | Height | The height of the floor from the previous floor as a numeric value in the unit type defined in the Cobie.Facility.LinearUnits for the project. |  |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Floor Schedule

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName

Space

Identify the following COBie Space information required to be delivered

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Space Nomenclature (format) Floor levelnn | 1013 |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | The space will be assigned a classification category from OmniClass using Spaces by function or form: |  |
| ☒ | FloorName | Reference to Floor Name field the space is located on | (GF) |
| ☒ | Description | A description of the space, room, or area. | (Main office) |
| ☐ | ExtSystem | The External System for the Space from which the data came. |  |
| ☐ | ExtObject | The External Object of the System for the Space from which the data came. |  |
| ☐ | ExtIdentifier | The External Identifier of the System for the Space from which the data came |  |
| ☒ | RoomTag | Room Tag Nomenclature (format) Cobie.Space.Name | (1013) |
| ☒ | UsableHeight | The usable height as a numeric value in the unit type as defined in the Cobie.Facility.LinearUnits for the project (Use '0' for unusable rooms). | (3000) |
| ☐ | GrossArea | The gross measured area as a numeric value in the unit type defined in the Cobie.Facility.AreaUnits for the project. | (245) |
| ☒ | NetArea | The net measured area, (subtract all columns and boxing areas) as a numeric value in the unit type defined in the Cobie.Facility.AreaUnits for the project. | (240) |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Room Data Sheets, Room Photos

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName

Zone

Identify the following COBie Zone information required to be delivered:

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Default Zone the Name of the Zone type | (Circulation) |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | The zone will be assigned with this classification category: OmniClass Spaces by Function. | (13-25 00 00) |
| ☒ | SpaceNames | Comma separated list of Space Names from Cobie.Space.Name | (1010,1011,1012) |
| ☒ | ExtSystem | The External System for the Zone from which the data came. |  |
| ☐ | ExtObject | The External Object of the System for the Zone from which the data came. |  |
| ☐ | ExtIdentifier | The External Identifier of the System for the Zone from which the data came |  |
| ☒ | Description | Full written description of purpose of zone | (Main Circulation) |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

zone strategy

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName

Asset Type

Identify the following COBie Facility information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Nomenclature (format) XXXXXX_nn | 210223 |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | The product will be assigned a classification category from OmniClass Table 21 | (21-02 10 80) |
| ☒ | Description | A description of the product (Manufacturer’s summary of the product) | (Spiral Stairs deluxe) |
| ☒ | AssetType | Is the product fixed or movable? | (Fixed) |
| ☒ | Manufacturer | The manufacturer of the product and provided as the Cobie.Contact.Email of the manufacturer record. | (Stair Man) |
| ☒ | ModelNumber | The manufacturers grouping or range name for the product. | (Spiral Special) |
| ☒ | WarrantyGuarantorParts | The Cobie.Contact.Email of the organisation responsible for supplying any parts under the warranty | (abc@mail.com) |
| ☒ | WarrantyDurationParts | The unit of time expressed as a number for which the parts are under warranty | (5) |
| ☒ | WarrantyGuarantorLabor | The Cobie.Contact of the organisation responsible for replacing or fixing the parts under the warranty | (abc@mail.com) |
| ☒ | WarrantyDurationLabor | The unit of time expressed as a number for which the labour will be supplied under the warranty. | (1) |
| ☒ | WarrantyDurationUnit | The unit type the warranty duration of parts and labour are expressed. | (years) |
| ☒ | ExtSystem | The External System for the Asset Type from which the data came. |  |
| ☒ | ExtObject | The External Object of the System for the Asset Type from which the data came. |  |
| ☒ | ExtIdentifier | The External Identifier of the System for the Asset Type from which the data came |  |
| ☒ | ReplacementCost | Cost to replace / purchase price | (90) |
| ☒ | ExpectedLife | The expected life of the product as implemented in the project expressed as a number of the type of duration unit | (25) |
| ☒ | DurationUnit | The expected life duration unit type expressed as a unit of time or operating cycle | (Years) |
| ☒ | WarrantyDescription | A Description of the warranty terms for the product | (Onsite warranty) |
| ☒ | NominalLength | Nominal Length size value that defined the extents of the product | (5000) |
| ☒ | NominalWidth | Nominal Width size value that defined the extents of the product | (2100) |
| ☒ | NominalHeight | Nominal Height size value that defined the extents of the product | (6000) |
| ☒ | ModelReference | The manufacturers grouping or range name for the product. |  |
| ☒ | Shape | An adjective or short description of the product shape. |  |
| ☒ | Size | A descriptive size provided by the manufacturer |  |
| ☒ | Color | The primary colour or colours of the product | (white) |
| ☒ | Finish | The primary finish or finishes of the product | (matt) |
| ☒ | Grade | The Grade quality of the product |  |
| ☒ | Material | The main materials the product is constructed from. | (metal) |
| ☒ | Constituents | If the product is in more than one part, then the parts should be lists. e.g.: a split Air conditioner will be indoor unit (evaporator), outdoor unit (Condenser) and remote control. | Main stair |
| ☒ | Features | Any features of the product that are not already included in the product description. |  |
| ☒ | AccessibilityPerformance | Any certification or accreditation that product has in relation to accessibility. | visual impairment adapted |
| ☒ | CodePerformance | Any certification or accreditation that product has in relation to accessibility. |  |
| ☒ | SustainabilityPerformance | Any certification or accreditation that product has in relation to quality codes. |  |
| ☐ | Length | The length of the product from the manufacturer in the units defined in Cobie.Facility.LinearUnits |  |
| ☐ | Area | The area of the product from the manufacturer in the units defined in Cobie.Facility.AreaUnits |  |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Product Data Sheet, COSH Sheet

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Asset Component

Identify the following COBie Asset Component information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Nomenclature (format) Cobie.Type.Name_ unique mark | 6162672872 |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | TypeName | Nomenclature (format) TYPE_nn | (Door_I4P_1100x2110_C) |
| ☒ | Space | The name of the Space the instance of the product occupied which must exist in Cobie.Space.Name. Where the product occupied multiple spaces, these are to be listed with the “,” comma separator. | 1011, 1012 |
| ☒ | Description | Description of the Asset Instance is different for the Asset Type | Fire Door |
| ☒ | ExtSystem | The External System for the Asset from which the data came. |  |
| ☒ | ExtObject | The External Object of the System for the Asset from which the data came. |  |
| ☒ | ExtIdentifier | The External Identifier of the System for the Asset from which the data came |  |
| ☒ | SerialNumber | If the product is designed and supplied for the project with unique serial numbers to identify it to the manufacturer. | 4256 |
| ☒ | InstallationDate | The date the product instance is installed Format UTC YYYY-MM-DD | 41640 |
| ☒ | WarrantyStartDate | The date the product warranty starts. If Warranty in Cobie.Type is required, then this should be checked. Format UTC YYYY-MM-DD |  |
| ☒ | TagNumber | AIR Tag Number (format) Wnnnn The Tag Number for the instance of the asset must be unique and should be the same as that detailed in drawings and schedules |  |
| ☒ | BarCode | If the product instances are being bar coded for the building owner during or post installation. | 8343 |
| ☒ | AssetIdentifier | The Asset Identifier provided by the Building Owner to be recorded by the installer. | 000004_SA/127/G/FI/DE/008343 |
| ☒ | Length | The length of the product installed in the units defined in Cobie.Facility.LinearUnits | 1010 |
| ☒ | Area | The area of the product installed in the units defined in Cobie.Facility.AreaUnits | 1.1 |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Test Certificate, Asset Register, Product Photo

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Asset System

Identify the following COBie Asset System information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Unique Name of the System | 141312331 |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | CobieSystemCategory The system will be assigned with this classification category. | Electrical |
| ☒ | ComponentNames | List of Asset Instances (Components) comma separated that belong to the system. | 6162672872, 3131231332 |
| ☒ | ExtSystem | The External System for the System from which the data came. |  |
| ☒ | ExtObject | The External Object of the System for the System from which the data came. |  |
| ☒ | ExtIdentifier | The External Identifier of the System for the System from which the data came |  |
| ☐ | Description | A description of the system. | Internal/General Lighting |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

System Register, Test Certificates

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Assembly

Identify the following COBie Facility information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Unique numerical assembly number | 321321313 |
| ☒ | CreatedBy | Supplier who created Record | (abc@mail.com) |
| ☒ | CreatedOn | Date record was created | (2020‐01‐15T14:30:00) |
| ☒ | AssemblyType | The type of relationship between the assembled assets | Layer |
| ☒ | SheetName | Related table of the assembled assets | Type |
| ☒ | ParentName | Reference to related table ‐ Name | 1232131421 |
| ☒ | ChildNames | Reference to related table ‐ Name | 1422441242 |
| ☒ | ExtSystem | Blank or n/a only |  |
| ☒ | ExtObject | Blank or n/a only |  |
| ☒ | ExtIdentifier | Blank or n/a only |  |
| ☒ | Description | Plain text description of the purpose of the assembly | Floor build up |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Test Certificates, Product Data, Product Photo

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Spare

Identify the following COBie Facility information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | A unique key name for each spare record suggests Cobie.Type.Name_nn | (12345678) |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | Type of spare |  |
| ☒ | TypeName | Cobie.Type.Name for which the spare is part of. |  |
| ☒ | Suppliers | The supplier of the spare which exists in the as a Cobie.Contact.Email record. |  |
| ☐ | ExtSystem | The External System for the Space from which the data came. |  |
| ☐ | ExtObject | The External Object of the System for the Space from which the data came. |  |
| ☐ | ExtIdentifier | The External Identifier of the System for the Space from which the data came |  |
| ☒ | Description | Description of the spare | (Projection bulbs) |
| ☒ | SetNumber | If the item is part of a set the reference of that set (similar to the Part Number) | (587-1) |
| ☒ | PartNumber | Manufacturer’s part number | (587) |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Spares Policy, Product Photo

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Resource

Identify the following COBie Resource information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Unique name for the resource | (White Board USB) |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | Type of resource, person, tools, materials |  |
| ☒ | ExtSystem | The External System for the Resource from which the data came. |  |
| ☒ | ExtObject | The External Object of the System for the Resource from which the data came. |  |
| ☒ | ExtIdentifier | The External Identifier of the System for the Resource from which the data came | (Projection bulbs) |
| ☒ | Description | Description of the resource | (Manufacturer’s upgrade memory stick) |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Test Certificates

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Job

Any maintenance required by the manufacturer or the installation contractor to ensure product reliability and performance to be detailed in the Cobie.Job with task, frequency, resources, and anticipated time for the task. Identify the following COBie Job information required for delivery.

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Unique name for the Record | (White Board USB) |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | See recommended option |  |
| ☒ | Status | The status of the task. Not Yet Started, Started, Completed |  |
| ☒ | TypeName | Cobie.Type.Name |  |
| ☒ | Description | Description of the Task |  |
| ☒ | Duration | The duration of the task expressed as a number |  |
| ☒ | DurationUnit | The unit type of the duration from the recommend list of time, frequency units |  |
| ☒ | Start | The date/time the first time this task is to be implemented or has been implemented. YYYY-MM-DDThh:mm:ss |  |
| ☒ | TaskStartUnit | The task start unit year, month, week |  |
| ☒ | Frequency | The frequency this task is to be performed expressed as a number |  |
| ☒ | FrequencyUnit | The unit type of the duration from the recommend list of time, frequency units |  |
| ☒ | ExtSystem | The External System for the Job from which the data came. |  |
| ☒ | ExtObject | The External Object of the System for the Job from which the data came. |  |
| ☒ | ExtIdentifier | The External Identifier of the System for the Job from which the data came |  |
| ☒ | TaskNumber | If this task is part of a sequence of tasks, then express this as numerical ordering. |  |
| ☒ | Priors | The task TaskNumber that precedes this task | () |
| ☒ | ResourceNames | A comma separated list of the resources, COBie.Resource.Name | () |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Test Certificates

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Document

Identify the following COBie Document information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☒ | Name | Unique Name for the Document | (White Board USB) |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | Document Type as defined in the definition |  |
| ☒ | ApprovalBy | Cobie.Contact.Email of the approver of the document |  |
| ☒ | Stage | See Stages below |  |
| ☒ | SheetName | Reference Sheet Name |  |
| ☒ | RowName | Reference object Name from the Reference Sheet |  |
| ☐ | Directory | The relative path of the folder for the document |  |
| ☒ | File | File Name of the document |  |
| ☒ | ExtSystem | The External System for the Space from which the data came. |  |
| ☒ | ExtObject | The External Object of the System for the Space from which the data came. |  |
| ☒ | ExtIdentifier | The External Identifier of the System for the Space from which the data came |  |
| ☒ | Description | Description on the contents of the document |  |
| ☒ | Reference | Name of the document from which this is a part if applicable |  |

The following documents are required and to be defined in the Cobie.Document table.

Document Types:

- Preconstruction Submittals

- Shop Drawings

- Product Data

- Product Photographs

- Samples

- Design Data

- Test Reports

- Test Certificates

- Certificates

- Manufacturer Instructions

- Manufacturer Field Reports

- Operation and Maintenance

- Closeout Submittals

- Contract Drawings

- Design Review Comment

- Specifications

- Request for Information

- Appointing Party / Client Requirements

- Contract Specifications

- Contract Drawings

- Requests for Information

- Contract Modifications

- Punch List Items

Additional attributes required as defined by any attached Template where attributes have been selected from:

TemplateName or schedule of product data templates

Attribute

Identify the following COBie Attribute information required for delivery:

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
| ☐ | Name | Attribute Name refer to each section as to the Attribute Names | (Weight) |
| ☒ | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
| ☒ | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
| ☒ | Category | See List of Attribute Categories per Sheets | (Submitted) |
| ☒ | SheetName | Reference Sheet Name | (Type) |
| ☒ | RowName | Reference object Name from Reference Sheet |  |
| ☒ | Value | Attribute value | (35.000) |
| ☒ | Unit | Unit notation of the attribute value. | (kg) |
| ☒ | ExtSystem | The External System for the Attribute from which the data came. |  |
| ☒ | ExtObject | The External Object of the System for the Attribute from which the data came. This should represent the Property Set grouping for the Attribute. |  |
| ☒ | ExtIdentifier | The External Identifier of the System for the Attribute from which the data came. | (n/a) |
| ☒ | Description | Description of the Attribute | (Installed Weight) |
| ☒ | AllowedValues | Permitted Values if relevant are only required in manually edited Cobie Workbooks | (n/a) |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Test Certificates

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Issue

Identify the following COBie Facility information required for delivery

| Req. | Attribute | Origin / Description | Example |
| --- | --- | --- | --- |
|  | Name | Attribute Name refer to each section as to the Attribute Names | (Weight) |
|  | CreatedBy | Cobie.Contact.Email of the author of the record | (abc@mail.com) |
|  | CreatedOn | UTC Date Time format when the record was created or changed YYYY-MM-DDThh:mm:ss | (2020‐01‐15T14:30:00) |
|  | Type | See List of Attribute Categories per Sheets | (Submitted) |
|  | Risk | Identified Risk associated to the issue | (High) |
|  | Chance | Chance of Issue occurring | (Has Occurred) |
|  | Impact | Possible Impact of Issue | (High) |
|  | SheetName1 | Reference Sheet 1 Name | (Document) |
|  | RowName1 | Reference object 1 Name from Reference Sheet | (Safety Policy) |
|  | SheetName2 | Reference Sheet 2 Name | (Component) |
|  | RowName2 | Reference object 2 Name from Reference Sheet | (n/a) |
|  | Description | Description of the Issue | (Board:12345) |
|  | Owner | Cobie.Contact.Email of the owner of the Issue | (abc@mail.com) |
|  | Mitigation | Mitigation associated to the Issue | (Warnings to Operatives) |
|  | ExtSystem | The External System for the Issue from which the data came. | (Revit) |
|  | ExtObject | The External Object of the Issue for the Attribute from which the data came. This should represent the Property Set grouping for the Attribute. | (Pset_Risk) |
|  | ExtIdentifier | The External Identifier of the Issue for the Attribute from which the data came. | (n/a) |

The following documents are required and to be defined in the Cobie.Document table.

Document Types

Test Certificates

Additional attributes required as defined by any attached Template where attributes have been selected from: TemplateName or schedule of product data templates

Delivery of COBie

The delivery of COBie data will follow use the following methodology across the project to aid in consistent data delivery:

(Client may wish to indicate their preferred methodology or leave to the delivery team.)

| Software | Document |
| --- | --- |
| Autodesk Revit | BIM interoperability tool will be used from all Revit models |
| Industry Foundation Classes | Solibri will be used to generate COBie from IFC files |

Further COBie Information

Further information regarding the use and application of COBie can be found from:

BuildingSMART Resources for COBie Certified professionals:

https://cobie.buildingsmart.org/resources/downloads/

BS 1192-4:2014 - Collaborative production of information. Fulfilling employer’s information exchange requirements using COBie. Code of practice.